Testing Schedule for garage door opening program

The Zelio should be wired to the testing rig accordingly

| Inputs Table |  |  |
| --- | --- | --- |
| Button/Switch label | Physical button | PLC connection |
| Open Door (Up) Button | Normally Open (NO) | I1 |
| Bottom Limit Switch (LS2) | Normally Open (NO) | I2 |
| Top Limit Switch (LS1) | Normally Open (NO) | I3 |
| Close Door (Down) Button | Normally Open (NO) | I4 |
| Stop Button | Normally Closed (NC) | IA |
| Outputs Table |  |  |
| Output device label | Physical device | PLC connection |
| Up LED | LED indicating motor moving door up | QB |
| Down LED | LED indicating motor moving door down | QD |
| Motor | Motor goes on | QF |
| Buzzer | Buzzer | QG |

Demonstration will involve loading your PLC program into a Zelio work station and running the PLC program.

The following tests will be conducted in this order

- Pressing the up button I1 (NO) should result in outputs QB and GF going true.

- Pressing the stop button IA (NC) should result in outputs QB and GF going false.

- Pressing the down button I4 (NO) should result in outputs QD and GF going true.

- Pressing the stop button IA (NC) should result in outputs QD and GF going false.

- Pressing the up button I1 (NO) should result in outputs QB and GF going true.

- Activating top limit switch I3 (NO) within 20s should result in outputs QB and GF going false.

- Pressing the down button I4 (NO) should result in outputs QD and GF going true.

- Activating bottom limit switch I2 (NO) within 20s should result in outputs QD and GF going false.

- Pressing the up button I1 (NO) should result in outputs QB and GF going true.

- Waiting for 20s should result in outputs QB and GF going false and output QG going true

- Pressing the up button I1 (NO) should result in QG going false or pressing the down button I4 (NO) should result in QG going false.