| Carpentry | Rates | Supply | Cartage | Wastage | Fixings | Equipment | Total cost of materials | Labour constant | Total labour cost | TOTAL COST |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 x 50 top plate | 47.37 |  | 5% | 5% |  | - | 8.75 | 0.18 | 8.53 | m |
| Materials |  |  |  |  |  |  |  |  |  |  |
| 100 x 50 H1 |  | 6.13 |  |  |  |  |  |  |  |  |
| Fixings $200/kg allow 2kg/m3 |  |  |  |  |  |  |  |  |  |  |
| 2 x $200.00 x .1 x.05 |  |  |  |  | 2.00 |  |  |  |  |  |
| Material total |  | 6.13 | 0.31 | 0.31 | 2.00 |  |  |  |  |  |
| Labour |  |  |  |  |  |  |  |  |  |  |
| Carpentry gang rate |  |  |  |  |  |  |  |  |  |  |
| 100 x 50 top plate |  |  |  |  |  |  |  |  |  |  |
| TOTAL COST |  |  |  |  |  |  | 8.75 |  | 8.53 | $ 17.28 |